Robotics: Assignment III

Team 16

鄭達郁 B09507016, 邱奕翔 B09901058

王泓予 B09901060, 薛尚齊 B09902037

Part A: Camera Calibration

1. The camera we use

We use the Iphone 14’s main camera, which is 12MP, and has a 2.6 mm, f/1.5 aperture. Detailed information of images are listed below.

Fig 1. Camera information

2. Calibration result

- Size

The size of each square in the chessboard is  2.4cm * 2.4 cm .

- Intrinsic parameters (unit: mm):

Camera intrinsic parameters include the focal length and the optical center. The camera intrinsic matrix (unit: mm) we found is shown below.

Fig 2. Camera matrix of the camera we used

- Steps

First, we define the world coordinates for 3D points and also define arrays objpoints and imgpoints to store 3D and 2D points for each checkerboard corner respectively and then take photos on checkerboard from different orientations.

Applying findChessboardCorners function from OpenCV library, we may find corners of the checkerboard and return coordinates of them.

After above steps are done, we then do the camera calibration. Knowing all the 3D points and related 2D relations in each image, we can get

- Intrinsic camera matrix

- Lens distortion coefficients

- Rotational and translational vector that brings the calibration pattern from the object coordinate space to the camera coordinate space ( both specified as a 3×1 vector)

by calling calibrateCamera function in OpenCV library.

- Physical meaning of each component of the Intrinsic matrix:

K= fxγcx0fycy001

fx:  Focal length in x-direction

fy:  Focal length in y-direction

cx:  Optical center in image plane in x-direction

cy:  Optical center in image plane in y-direction

γ :  The skew between axis (usually equals to zero).

- Program task

Original checkerboard images are in calibrate_src folder, and the original 21st image is in input folder. The undistorted images will be generated into output folder after running the program.

Our program will show the original images with their corners marked and then undistorted images. The user may press any key to switch to the next image. The information we get from calibrateCamera method will be printed in console when the program gets to its end.

Fig 4. Undistorted 20 images

Fig 3. Original 20 images

Fig 6. Undistorted 21st image

Fig 5. Original 21st image

- Discussion

Some pin-hole camera induce radial dilation which makes the images distort. For example, we observed at the original checkerboard images and found some of the lines are distorted into curves in the checkerboard pattern. Applying camera calibration, we can find the camera’s intrinsic matrix and the relation of corners among different images. Using the relation we found, we may then undistort the image.

However, in reality it’s not that easy to observe the effect of the undistortion since the images we captured are only slightly distorted, but we still can tell the effect of transformation from the size of dark corners around the images when they are undistorted.

According to OpenCV documentation, Re-projection error gives a good estimation of just how exact the found parameters are, thus we use projectPoints method in OpenCV library to calculate the absolute norm between what we got with our transformation and the corner finding algorithm. Then find the arithmetical mean of the errors calculated for all the calibration images.

The re-projection mean error of these images are 0.0412, which is rather a small error, showing that the parameters we found are quite accurate.

Part B: Object Detection

In this part, we used opencv to process the images. The FindContours() function can find the contours of each object, but we have to preprocess the images before applying it. First, we used cvtColor(COLOR_BGR2GRAY) to turn the picture into grayscale. Then, we used GaussianBlur() to blur the image a bit so that the noise can be reduced thus increasing the accuracy. Also we make the images become binary images by using threshold(). After that, we can apply FindContours() and get the contours.

For each object detected, we can calculate the central moment with moments(). Then we can calculate the centroids and principle angles as taught in class. With line() and circle(), we can plot the centroids and moment of inertia on the images.

Division of work

- Image collecting: B09507016

- Coding: B09902037, B09901058

- Code review: B09507016, B09901060

- Report:

  - PartA: B09507016, B09901060

  - PartB: B09902037

- Report revision: B09901058

GitHub link

https://github.com/ZOIjimmy/Robotics/tree/master/AssignmentIII

References

[1]  https://docs.opencv.org/4.x/dc/dbb/tutorial_py_calibration.html